CSCD 240 Homework 3 and 4 (Project ) (Total 200 points)

Simple Image Processing using C pointer to pointers, string operations,

type conversion, math library, command-line parsing and file I/O

This is a comprehensive project, which is weighted twice as much as a regular homework. That is, the whole project accounts for 200 points totally. When calculating the weighted final score, this project is considered as two pieces of homework.

Description

In this project, we manipulate PGM images (Portable Gray Map). PGM files could be pure ASCII text files, with header information and intensity (brightness) value for each pixel in an image file. If you have an image named ballon.pgm, you can use command less ballon.pgm to explore its format. Of course, you can install an image viewer to visualize the image with your naked eye. On windows, you can use Irfanviw, and download is available at http://www.irfanview.com

On a Mac machine, you can download ToyViwer in your apple store for free.

The detailed format description for PGM file can be found here. (Also you can download more sample PGM files, besides one included in this project package.)

http://people.sc.fsu.edu/~jburkardt/data/pgma/pgma.html

The following is an example

P2

# feep.ascii.pgm

24 7

15

0 0  0  0  0  0  0  0  0 0  0  0  0  0  0  0  0 0  0  0  0  0  0  0

0 3  3  3  3  0  0  7  7 7  7  0  0 11 11 11 11 0  0 15 15 15 15  0

0 3  0  0  0  0  0  7  0 0  0  0  0 11  0  0  0 0  0 15  0  0 15  0

0 3  3  3  0  0  0  7  7 7  0  0  0 11 11 11  0 0  0 15 15 15 15  0

0 3  0  0  0  0  0  7  0 0  0  0  0 11  0  0  0 0  0 15  0  0  0  0

0 3  0  0  0  0  0  7  7 7  7  0  0 11 11 11 11 0  0 15  0  0  0  0

0 0  0  0  0  0  0  0  0 0  0  0  0  0  0  0  0 0  0  0  0  0  0  0

PGM file named smallFile.pgm

The image header consists of the first 4 lines.

- The first line P2, is a magic number to tell the image viewer that the file is a ASCII pgm file.

- The second line starts with #, is a line of comment.

- The numbers in the third line means this image has 24 COLUMNS, and 7 ROWs of pixels in it. Note: column goes first!

- The fourth line means the maximum intensity value in the image is 15. Each pixel has intensity value. The bigger intensity value is, the brighter or whiter at that location is in the image. The intensity value 0 means a total black color in the image.

Intensity values of all pixels are listed following the file header, starting with 5th line. These intensity values could be considered as a 2D array, with the row index and column index specifying each point (or pixel) at a particular location.

What you should do?

Please carefully read the comments on top of each function declared in pgmUtility.h file.

- In pgmUtility.c file, implement the function that reads in the image (actually a text file) using file I/O. The function has been declared already in pgmUtility.h file.

int ** pgmRead( char **header, int *numRows, int *numCols, FILE *in  );

- In pgmUtility.c file, implement the function that paints a black dot (circle) in the image, you have to parameterize the center point and the radius of the circle you will draw. The function has been declared already in pgmUtility.h file.

int pgmDrawCircle( int **pixels, int numRows, int numCols, int centerRow,

int centerCol, int radius, char **header );

- In pgmUtility.c file, implement the function that paints a black edge frame in the image, you have to parameterize the width of the edge you will paint. The function has been declared already in pgmUtility.h file.

int pgmDrawEdge( int **pixels, int numRows, int numCols, int edgeWidth, char **header );

- Note that after you paint the edge or the black dot in an image, you have to update the header to reflect the new maximum intensity value in the header (specifically in the last line of image header). NOTE: this operation might render the new image brighter when you compare with the original image.

- In pgmUtility.c file, implement the function that writes back to a new image file that contains your painting using file I/O. The function has been declared already in pgmUtility.h file.

int pgmWrite( const char **header, const int **pixels, int numRows, int numCols, FILE *out );

- You are required to define the functions that are provided in the *.h file, you CANNOT change the signatures (parameter list and return type) of these functions. You need to define it in its corresponding *.c file and use them in main function if necessary. The purpose of this *.h file is to help you understand how pointers are used in real world applications. Please carefully read the comments on top of each function declared in pgmUtility.h file.

- You are required to deallocate all memories you dynamically allocated in program. Please make sure you do it in a proper timing and at proper location in your program, where you know these memories are no longer useful. In addition, you are required to check your memory deallocation by running your program with valgrind command on cslinux machine. Please create a pdf file that shows the valgrind memory-check result.

- Add a simple makefile to compile your source code into an executable hw34.

- You have to write your own main() function, in which, if necessary, you call these function(s) specified above in order to output a new image file on disk.  Depending on the command-line arguments passed in, the new image should look like what is shown in the test cases section below.

- When you run your program, you have to pass in the Circle Center, Radius or Edge Width as command line arguments. In your main function, you have to write code to parse the command-line arguments. Also, if the number of command line argument is not expected, your program are required to show a message as below:

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

You have to run your program using command with this synopsis:

./programName –e edgeWidth  originalImage  newImageFile

in order to paint an edge of width of edgeWidth in the image of originalImage

./programName –c circleCenterRow circleCenterCol  radius  originalImage  newImageFile

in order to paint an big round dot on the image with center at (circleCenterRow, circleCenterCol) and radius of radius.

When an user inputs wrong number of argument in command line or in a wrong format, your program should not crash, instead showing the Usage message above.

If your program could handle drawing a circle and an edge at the same time in one command in your terminal window you get another 10 bonus points on top of 200.

In this case, your command line should look like the following; otherwise you cannot get the bonus.

./programName -ce circleCenterRow circleCenterCol radius edgeWidth      originalImage  newImageFile

AND

./programName -c -e circleCenterRow circleCenterCol  radius  edgeWidth      originalImage  newImageFile

Test cases with the provided image

./myPaint -c 470 355 100  ./balloons.ascii.pgm  balloons_c100_4.pgm

Your program yields an image looks like:

./myPaint -c 228 285 75  ./balloons.ascii.pgm  balloons_c75_5.pgm

The command above yields an image,

./myPaint -e 50  ./balloons.ascii.pgm  balloons_e50_2.pgm

The command above produces an image that looks like,

If input wrong command line parameters:

./myPaint -e 50 300  ./balloons.ascii.pgm

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

./myPaint -e ab  ./balloons.ascii.pgm

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

./myPaint -e   ./balloons.ascii.pgm

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

./myPaint -c 470 355  ./balloons.ascii.pgm

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

./myPaint -c 470 355 50 60  ./balloons.ascii.pgm

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

./myPaint -c 470 90bc   ./balloons.ascii.pgm

Usage:

-e edgeWidth  oldImageFile  newImageFile

-c circleCenterRow circleCenterCol radius  oldImageFile  newImageFile

The original image before your processing looks like:

To turn in:

Please wrap up all your source code, your pdf file that shows the valgrind check result and all included test images into a single zip file, name it as lastname  + firstinitial + h34.zip. If you are Will Smith, your zip file is named as smithwh34.zip.

If you did the bonus points part, please clearly state that on the top of your pdf file originally used for showing valgrind check result.

Turn in your single zip file on Canvas CSCD240 -> Assignments->hw34->submit

If your code shows a compile error or a segmentation fault, you will get zero credit. If your file is corrupted, you get a zero credit.